SSI External Management Specification (Dashboard)

September 2010

Revision 1.0.1

Legal Statement

- THE SERVER SYSTEM INFRASTRUCTURE PROMOTERS (“SSI PROMOTERS”) MAKE NO WARRANTIES WITH REGARD TOTHIS SSI SPECIFICATION (“SPECIFICATION”), AND IN PARTICULAR DOES NOT WARRANT OR REPRESENT THAT THIS SPECIFICATION OR ANY PRODUCTS MADE IN CONFORMANCE WITH IT WILL WORK IN THE INTENDED MANNER. NOR WILL SSI PROMOTERS ASSUME ANY RESPONSIBILITY FOR ANY ERRORS THAT THE SPECIFICATION MAY CONTAIN OR HAVE ANY LIABILITIES OR OBLIGATIONS FOR DAMAGES, INCLUDING BUT NOT LIMITED TO SPECIAL, INCIDENTAL, INDIRECT, PUNITIVE, OR CONSEQUENTIAL DAMAGES WHETHER ARISING FROM OR IN CONNECTION WITH THE USE OF THIS SPECIFICATION IN ANY WAY.

- NO REPRESENTATIONS OR WARRANTIES ARE MADE THAT ANY PRODUCT BASED INWHOLE OR PART ON THE ABOVE SPECIFICATION WILL BE FREE FROM DEFECTS OR SAFE FOR USE FOR ITS INTENDED PURPOSE. ANY PERSON MAKING, USING OR SELLING SUCH PRODUCT DOES SO AT HIS OR HER OWN RISK.

- THE USER OF THIS SPECIFICATION HEREBY EXPRESSLY ACKNOWLEDGES THAT THE SPECIFICATION IS PROVIDED AS IS, AND THAT THE SSI PROMOTERS MAKE NO REPRESENTATIONS, EXTENDS NO WARRANTIES OF ANY KIND EITHER EXPRESS OR IMPLIED ORAL OR WRITTEN, INCLUDING ANY WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE, OR WARRANTY OR REPRESENTATION THAT THE SPECIFICATION OR ANY PRODUCT OR TECHNOLOGY UTILIZING THE SPECIFICATION OR ANY SUBSET OF THE SPECIFICATION WILL BE FREE FROM ANY CLAIMS OF INFRINGEMENT OF INTELLECTUAL PROPERTY, INCLUDING PATENTS, COPYRIGHTS AND TRADE SECRETS NOR DO THE SSI PROMOTERS ASSUME ANY OTHER RESPONSIBILITIES WHATSOEVER WITH RESPECT TO THE SPECIFICATION OR SUCH PRODUCTS.

- A NON-EXCLUSIVE COPYRIGHT LICENSE IS HEREBY GRANTED TO REPRODUCE THIS SPECIFICATION FOR ANY PURPOSE PROVIDED THIS “IMPORTANT INFORMATION AND DISCLAIMERS SECTION (PARAGRAPHS 1-6) IS PROVIDED IN WHOLE.

- UPON REQUEST FROM AN ADOPTER, THE SSI PROMOTERS WILL GRANT A NON-EXCLUSIVE, WORLD-WIDE LICENSE UNDER ANY NECESSARY CLAIMS, DEFINED IN THE ADOPTERS AGREEMENT, TO MAKE, HAVE MADE, USE, IMPORT, SELL, OFFER TO SELL, AND OTHERWISE DISTRIBUTE AND DISPOSE OF COMPLIANT PORTIONS, DEFINED IN THE ADOPTERS AGREEMENT, PROVIDED THAT SUCH LICENSE NEED NOT EXTEND TO ANY PART OR FUNCTION OF A PRODUCT IN WHICH A COMPLIANT PORTION IS INCORPORATED THAT IS NOT ITSELF PART OF THE COMPLIANT PORTION. SUCH LICENSE WILL BE GRANTED ON REASONABLE AND NONDISCRIMINATORY (“RAND”) TERMS AND MAY BE CONDITIONED UPON ADOPTER’S GRANT OF A RECIPROCAL LICENSE TO THE SSI PROMOTERS.

- NO OTHER LICENSE, EXPRESS OR IMPLIED, BY ESTOPPEL OR OTHERWISE, TO ANY OTHER INTELLECTUAL PROPERTY RIGHTS IS GRANTED.

Revision 1.0.1

*Product names are trademarks, registered trademarks, or service marks of their respective owners.

- Contents

1	Introduction	1

1.1	Terms and Abbreviations	1

1.2	Reference Documents	3

2	Dashboard Architecture	4

2.1	Introduction	4

2.2	Sample UI	4

2.3	Class Model	5

2.3.1	Component Class	6

2.3.2	LogEntry Class	10

2.3.3	InfoElement Class	11

2.3.4	Action and ActionParam Classes	13

2.3.5	HealthElement and HealthState Classes	15

2.3.6	Sensor and SensorValue Class	16

2.4	Enumerated List Clarifications	21

2.4.1	DataType	21

2.4.2	SeverityLevel	22

3	SOAP Interface Conventions	23

3.1	Interfaces	23

3.2	Method Invocation	23

3.3	Composition AccessMethods	23

3.3.1	Single Instance	23

3.3.2	Multiple Aggregated Instances	24

3.3.3	Specific Access Methods	24

3.3.4	Filter Parameters	24

3.4	SOAP WSDL Locations	27

A.1	Component Method Calls	28

A.2	Aggregated Data	28

A.3	Actions	29

A.4	Composite Aggregated Data	31

- Figures

Figure 2-1 Sample Dashboard GUI	4

Figure 2-2 Class Hierarchy	5

- Tables

Table 1-1. Terms and Abbreviations	1

- Revision History

The following table lists the revision schedule based on revision number and development stage of the product.

| Revision | Project Document State | Date |
| --- | --- | --- |
| 1.0.0 | Initial public release. | 9/17/2009 |

Note: Not all revisions may be published.

# Introduction

This document specifies a lightweight interface to perform basic manageability control and status reporting of SSI-compliant Blade Systems.

## Terms and Abbreviations

Table 1-1 lists the terms and acronyms used in specific ways throughout this document:

Table 1-1. Terms and Abbreviations

| Term | Definition |
| --- | --- |
| blade | This is resource module that plugs into the blade chassis. A blade can provide many different types of resources to the chassis, including compute functions, storage capabilities, additional I/O interfaces and switching capabilities and special purpose capabilities. A blade can be a single-wide module (assumed) or a double-wide module, occupying two adjacent slots in the chassis. |
| Blade server | A system comprised of a chassis, chassis resources (power, cooling, Chassis Manager), compute blades, and communication (switch) blades. The chassis may contain additional modules, such as storage. |
| chassis | The mechanical enclosure that consists of the mid-plane, front boards, cooling devices, power supplies, etc. The chassis provides the interface to boards and consists of the guide rails, alignment, handle interface, face plate mounting hardware, and mid-plane interface. |
| chassis ground | A safety ground and earth return that is connected to the chassis metal and available to all PBAs. |
| Chassis Management Module (CMM) | Dedicated intelligent chassis module that hosts the Chassis Manager functionality. |
| Chassis Manager (CM) | Set of logical functions for hardware management of the chassis. This may be implemented by one or more dedicated Chassis Management Modules or by one or more blade management controllers and/or payload processors. |
| handle | An item or part used to insert or extract blades into and out of chassis. |
| Intelligent Platform Management Bus (IPMB) | IPMB is an I2C-based bus that provides a standardized interconnection between managed modules within a chassis. ftp://download.intel.com/design/servers/ipmi/ipmb1010ltd.pdf |
| Intelligent Platform Management Interface (IPMI) | IPMI v2.0 R1.0 specification defines a standardized, abstracted interface to the platform management subsystem of a computer system. ftp://download.intel.com/design/servers/ipmi/IPMIv2_0rev1_0.pdf |
| Management Controller | An intelligent embedded microcontroller that provides management functionality for a blade or other chassis module. |
| may | Indicates flexibility of choice with no implied preference. |
| mezzanine | The mezzanine is a PBA that installs on a blade PBA horizontally. It provides additional functionality on the blade PBA and provides electrical interface between the blade PBA and the mid-plane PBA. Both the blade PBA and mezzanine PBA are contained inside the blade module. |
| mid-plane | Equivalent to a system backplane. This is a PBA that provides the common electrical interface for each blade in the chassis and on both the front and back of the PBA. |
| module | A physically separate chassis component that may be independently replaceable (e.g., a blade or cooling module) or attached to some other component (e.g., a mezzanine board). |
| out-of-band (OOB) | Communication between blades that does not need the host or payload to be powered on |
| payload | The hardware on a blade that implements the main mission function of the blade. On a compute blade, this includes the main processors, memory, and I/O interfaces. The payload is powered separately from the blade management subsystem. Payload power is controlled by the blade management controller. |
| Primary Channels | The 2 Baseboard Ethernet connections that every server blade must have that goes to the Primary Switch(es). These primary Ethernet channels are also shared on the blade for management traffic. |
| Primary Switch | The switch connected to the primary channels |
| SAP | Service Applet Package |
| shall | Indicates a mandatory requirement. Designers must implement such mandatory requirements to ensure interchangeability and to claim conformance with this specification. The use of shall not (in bold) indicates an action or implementation that is prohibited. |
| should | Indicates flexibility of choice with a strongly preferred implementation. The use of should not (in bold text) indicates flexibility of choice with a strong preference that the choice or implementation be avoided. |
| slot | A slot defines the position of one blade in a chassis |

## Reference Documents

- W3C Soap 1.2 Specification
http://www.w3.org/TR/soap12/

- JavaScript Object Notation (JSON)
RFC 4627: http://www.ietf.org/rfc/rfc4627.txt
http://www.json.org/

- DMTF Sensors Profile
http://www.dmtf.org/standards/published_documents/DSP1009_1.0.1.pdf

# Dashboard Architecture

## Introduction

The SSI Dashboard Service is intended to be hosted and exposed by a Chassis Management Module (CMM) on an SSI Blade System as a simple interface that will give easy access to the most commonly used features in a Blade System.

In general, the data structures are arranged in the form of a tree of nested components. Each component can have Log Entries, Info Elements (key/value pairs), Actions, Health Status, and Sensors. Additionally, the Components allow direct access to power control and to operational and health status without having to traverse many sub-classes, etc.

## Sample UI

With this structure, one could implement a basic dashboard user interface similar to the concept shown in Figure 2-1 without making too many calls to the backend server.

Figure 2-1 Sample Dashboard GUI

## Class Model

The top level “component” in the hierarchy is the “Component” Class (Figure 2-2).  All other classes are aggregated (directly or indirectly) under Component. The Component Class even aggregates itself. This allows a container Component (like a Blade Chassis, or Rack) to contain other Components (such as Blade Servers).  There shall exist only a single top (ROOT) Component object and its InstanceID shall be ROOT.

Figure 2-2 Class Hierarchy

In an SSI blade system, in addition to the ROOT component, a component object shall exist for the Chassis and each Compute Blade, Switch, IO Device, Power Supply, Fan, and active Manageable Components in the system. Each method as described herein shall be implemented as described in the detail below.

In an SSI Blade System, all components that are contained in a chassis shall be modeled under (either as sub-Components or lower as appropriate) the Chassis Component, which shall be of ComponentType="chassis". If no other components are modeled outside the chassis, then the Chassis Component shall be the ROOT component and have an InstanceID of ROOT.

### Component Class

The Component Class is the top level class that would be a close parallel to Managed Element from CIM. All Manageable Entities are Components.

Although there is an Action class aggregated by Component, common power control actions are methods that have been added as direct methods into the Component class.

Component Properties

| Property | Type | Notes |
| --- | --- | --- |
| InstanceID | string | This shall be a system-wide unique string (such as a GUID). |
| ComponentType | ComponentType | This represents a physical type of component: Chassis, Server, Fan, etc. Possible Values as Defined in <<enumeration>> ComponentType: unknown = 0, location = 1, room = 2, container = 3, rack = 4, chassis = 5, server = 6, drive = 7, io = 8, ps = 9, fan = 10, other = 11 |
| ComponentTypeDescription | string | Additional, more precise description for the component type. Example: a blade may be of type "Server" but a more specific description may be "Blade Server" |
| Caption | string | User friendly name of Component. *Example: "Server 1" |
| LocatorURL | string | Uniform Resource Locator (as defined by RFC 1378 - http://www.ietf.org/rfc/rfc1738.txt ) for the purpose of directly accessing the Component. If direct access is unavailable, but access to a higher level Component in the Hierarchy is available, then the URL to the higher level Component shall be entered. The intent is for this address to be launchable by a browser, and the URL shall use either http or https protocol. * Example: http://my_system/show_component?server1 |
| HealthState | HealthState | Health Status rollup indicator for this Component. This represents the total health state of the component. Additionally, a composition of 1 to many HealthElements are available as Health "sub-statuses," if additional granularity is available. Also, HealthStatus is intended to logically rollup the state of all health related Sensors to give an interpretation of the Health State of the Component. * See HealthState Class Definition |
| OperationalStatus | OperationalStatus | The Operational Status of Component represents its administrative or power state: On, Off, etc. This is not to be confused with the Health Status of the Component. Possible Values as Defined in <<enumeration>> OperationalStatus: off = 0, on = 1, suspended = 2, na = 3, unknown = 4, custom = 5 |
| OperationalStatusDescription | string | Friendly (and more detailed) description of the current operational status. This property should be used to provide clarity to the Operational Status. If Operational Status is "custom", then this field shall be filled. * Examples: On, Off, On/Rebuilding, On/Shutting Down |
| PowerControlCapabilities | string[] | This array defines what power control capabilities the Component has. Based on this list, the power control methods to the ComponentClass will be available for invocation. In some components (like a container) the array may be empty. Valid array members are: start, stop, reset, powercycle, gracefulshutdown, identify |
| TypeOrdinal | uint | Ordered Number identifying which order the component is of the given type. Example: For Blade 1, TypeOrdinal would be 1. For Switch, 1 the TypeOrdinal would be 1. For Power Supply 4, the TypeOrdinal would be 4. |
| IdentifyState | bool | Status of "identify" mechanism. The boolean values are: true = on false = off or n/a. |

Component Methods

| Method | Type | Notes |
| --- | --- | --- |
| invokeAction ( ActionName:string, ActionParameters:string) | short | Invokes a named Action from the Action list, particularly if an action is added that is not part of the standard power control actions methods built into the Component class. Parameters: ActionName [ string - in ]: Name of Action as defined in the "Name" property of the Action class. ActionParameters [ string - in ] : JSON encoded string of name value pairs. ActionParam::Name property shall be used as the name for the value. Example: '{"user": "asdf@asdf.com", "pass": "87h87yh8", "maxRetry": 3}' Returns: 0 for success; Positive Number as an Error Code |
| gracefulShutdown () | short | Performs a graceful shutdown of component. On an ACPI-supported system, a graceful shutdown shall imply an S5 transition (soft power off). Returns: 0 for success; Positive Number as an Error Code |
| powerCycle () | short | Invokes a power cycle algorithm on the component. This is typically a full power off (for several seconds) followed by a full power on. It differs from reset in that reset typically does not remove power to the platform. Returns: 0 for success; Positive Number as an Error Code |
| reset () | short | Resets component. This is also known as a "hard reset" Returns: 0 for success; Positive Number as an Error Code |
| start () | short | Start will power on the Component. If the component is already on, this method will have no effect. Returns: 0 for success; Positive Number as an Error Code |
| stop () | short | Stop will power off a component. If the power is already off, this method will have no effect. Care should be taken to ensure not to invoke this method at a lower level if the Component is already in transition to off. Returns: 0 for success; Positive Number as an Error Code |
| identify(Mode: uint) | short | Invokes visible identification mechanism on component. This is typically tied to a special identification LED or blinking sequence. Parameters: Mode [ uint – in ]: Set Mode of identification mechanism 0: Off 1: On (indefinitely) x: On x for seconds Returns: 0 for success; Positive Number as an Error Code |
| getSensorValues( SensorNames:string[]) | SensorValue[] | Retrieve Sensor Values (Name, Reading, State, Severity) that correspond to the array of sensor names sent. This special call is intended to refresh sensor values that were already retrieved by getSensorInstances(). Parameters: SensorNames [ string[] – in ]: Array of Sensor Names (as defined in the Sensor Class) Returns: Array of SensorValue |

### LogEntry Class

LogEntry objects shall exist for each log entry in the system. These are aggregated by the component to which the log entry belongs. Log Entries of any Component shall include all entries of any sub-Components, therefore, the Log Entries of the ROOT component shall be all Log Entries of the system. Log entries shall be returned, sorted by the (descending) DateTime field with the most recent entry first.

LogEntry Properties

| Property | Type | Notes |
| --- | --- | --- |
| InstanceID | string | This shall be a system-wide unique string (such as a GUID). |
| Caption | string | Short description of the entry as would be appropriate to display in a relatively small list. The verbose description should be in the "Description" property. |
| Closed | bool | This is an Indicator that the entry has been acknowledged. This is typically set by an end-user, and a typical use case would be to filter only the non-Closed (unacknowledged) Log Entries. |
| DateTime | ulong | Timestamp of when event occured. This is encoded in the convention of the "Unix Timestamp" (seconds since Jan 1, 1970). |
| Description | string | Long verbose description of the Log Entry |
| Severity | SeverityLevel | Possible Values as Defined in <<enumeration>> SeverityLevel * see 2.4.2 |
| MetaDataType | DataType | The type of data that exists in the Value property Possible Values as Defined in <<enumeration>> DataType: * see 2.4.1 |
| MetaData | string | Additional Encapsulated Data pertaining to this log entry. This field shall be encoded per the DataType. |

LogEntry Methods

| Method | Type | Notes |
| --- | --- | --- |
| close () | ushort | Sets Closed property to true. |

### InfoElement Class

Info Elements are used to provide key/value component data. All general tables of data that are not modeled elsewhere may be stored here. This is a good place for Product/Model, Serial Numbers, etc.

InfoElement Properties

| Property | Type | Notes |
| --- | --- | --- |
| DataType | DataType | The type of data that exists in the Value property. Possible Values as Defined in <<enumeration>> DataType: * see 2.4.1 |
| InfoType | InfoType | General meaning of data contained in the Value. This more represents how the data may be used rather than how it is encoded. Multiple InfoElements of a Component may be of the same InfoType. Possible values defined in <<enumeration>> InfoType The following InfoTypes shall be encoded with the defined <<enumeration>> DataTypes indicated in the table below. Value; Description; DataType. swversion = 0; Software Version; string. hwversion = 1; Hardware Version; string. vendor = 2; Vendor ID/Manufacturer; string. productmodel = 3; Product/Model Number; string. productname = 3; Product Name; string. mfgdate = 4; Manufacture Date; datetime. serial = 5; Serial Number; string. hostip = 7; Host IP Address; ipaddr. hostmac = 8; Host MAC Address; macaddr. ipmifru = 9; IPMI FRU Data; blob. other = 255; Other InfoType; <any>. |
| Caption | string | This is the string that is displayed as the label for the data. It may also be thought of as a Friendly Name in that it is free form and may contain spaces and other characters. It differs from Name in that Name is intended to represent the variable name behind the data that would include the restrictions required on a variable or field name (no spaces, strange characters, etc). All InfoElements that belong to a Component shall have unique captions. |
| Name | string | Variable name representing the data within the system. This string shall start with a letter and shall have no spaces or characters other than letters numbers and underscores. All InfoElements that belong to a Component shall have unique captions. |
| Description | string | Long description explaining item. This may be used as additional "help text". |
| Value | string | The InfoElement value encoded per DataType. |

### Action and ActionParam Classes

This structure is provided to add additional Component actions that go beyond what is defined in the Component methods.

Any number of additional actions can be defined for a Component that can be exposed in an external user interface. This definition allows for simple custom action invocation, or more complex ones that also need to gather additional defined input from the end user to pass to the invoked action.

The Action instances shall always be returned with ActionParam[] array pre-populated with all ActionParam instances.

Action Properties

| Property | Type | Notes |
| --- | --- | --- |
| InstanceID | string | This shall be a system-wide unique string (such as a GUID). |
| HelpText | string | Detailed help text for this action. In a UI, it could show up in a hover-over or help box. |
| Caption | string | Short description (or label) for this Action. In a user interface, this word will be what Identifies this action (it may even be displayed on a button). |
| Name | string | Name that is a "variable name" identifier for this Action. It carries the same restrictions as typical languages in variable names (no spaces, special characters, etc). |
| ActionType | ActionType | The general type of action this is. Although any type of action can be implemented, this gives the system (particularly in the case of automation), a hint as to what the action really does. start = 1 stop = 2 reset = 3 powercycle = 4 launchurl = 5 dialog = 6 other = 7 |
| URLInfo | string | If ActionType is launchurl, this is the URL that is to be launched. If the type is "launchurl" and non-Hidden ActionParams exist, once additional ActionParams are gathered from the end user, the URL shall be launched with ActionName="<Action::Name>" and each "ActionParameters[<ActionParam::Name>]=<value> sent as an HTTP POST to the URL. |

ActionParam Properties

| Property | Type | Notes |
| --- | --- | --- |
| DataType | DataType | The type of data that ActionParam is supposed to collect. Possible Values as Defined in <<enumeration>> DataType: * see 2.4.1 |
| Default | string | Parameter Default value |
| DisplayType | DisplayType | If this parameter is prompted in a UI, this gives a hint as to the display type of this parameter: Hidden = 0, Input = 1, Select = 2, Checkbox = 3, Radio = 4, TextArea = 5 These types actually imply certain other constraints: Hidden: Embedded Value (in the Default) Input: Free form textual or numeric input Select: Single value from a specific list of options Checkbox: Multiple values from a specific list of options Radio: Similar to select -- Single value from a specific list of options (except they are all visible at once) TextArea: Free form multi-line text |
| Caption | string | Label for this parameter |
| Name | string | Name that is a "variable name" identifier for this Parameter. It carries the same restrictions as typical languages in variable names (no spaces, special characters, etc). This is passed to the Component::invokeAction Parameters string. |
| OptionPrompts | string | In the case where DisplayType is Select, Checkbox, or Radio, these are the Prompts (Labels) for the available values. This is a comma delimited list. |
| OptionValues | string | In the case where DisplayType is Select, Checkbox, or Radio, these are the values that are associated with the prompts. This is a comma delimited list |
| HelpText | string | Detailed help text for this action. In a UI, it could show up in a hover-over or help box. |

### HealthElement and HealthState Classes

The intent of the HealthElement Aggregation from component is to allow the Component to provide a comprehensive list of health issues that may exist with any subcomponents. For example, although a Chassis may be healthy as a container, it may contain Components (such as blades) that are in a Critical State. This one-to-many aggregation gives an opportunity for the Container to "roll up" the health states and properly represent the Health as a system.

HealthElement Attributes

| Attribute | Type | Notes |
| --- | --- | --- |
| ComponentId | string | Component::ComponentId that the HealthElement represents. This does not have to be the aggregating Component. This could be the ID of a subComponent. |
| HealthStatus | HealthState | The HealthStatus indicator for the Component represented by ComponentId * See HealthState Class Definition |

HealthState Attributes

| Attribute | Type | Notes |
| --- | --- | --- |
| Severity | SeverityLevel | Possible Values as Defined in <<enumeration>> SeverityLevel * see 2.4.2 |
| Caption | string | Short description or label of this state |
| Description | string | Detailed description of status |
| RecommendedAction | string | Recommended action to take for this problem (if one exists) for the health state. This can optionally be left blank, particularly if the status is "Ok". |

### Sensor and SensorValue Class

The Sensor class is based on the DMTF CIM_NumericSensor class, and it is a full sensor representation including all information about how to interpret the Sensor. It is essentially a combined IPMI SDR and sensor value in one. Once a Sensor is retrieved, typically, only the changing values need to be updated. For this purpose, the SensorValue class exists as a proper subset of Sensor that only contains the transient data. The Sensor Attributes definition below contains the proper description to the fields in the SensorValue class. The Component::getSensorValues() method is used to update the sensor values.

Sensor Attributes

| Attribute | Type | Notes |
| --- | --- | --- |
| SensorName | string | Unique Name within the domain of a Component Instance |
| Severity | SeverityLevel | Possible Values as Defined in <<enumeration>> SeverityLevel * see 2.4.2 |
| Caption | string | Short friendly description of the Sensor |
| Accuracy | int | Indicates the accuracy of the Sensor for the measured property. Its value is recorded as plus/minus hundredths of a percent. Accuracy, along with Resolution, is used to calculate the actual value of the measured physical property. Accuracy may vary depending on whether the Device is linear over its dynamic range. Units: Hundredths of a Percent |
| SensorType | ushort | The Type of the Sensor, e.g. Voltage or Temperature Sensor. If the type is set to "Other", then the OtherSensorType Description can be used to further identify the type. Or, if the Sensor has numeric readings, then the type of the Sensor can be implicitly determined by the Units. A description of the different Sensor types is as follows: A Temperature Sensor measures the environmental temperature. Voltage and Current Sensors measure electrical voltage and current readings. A Tachometer measures speed/revolutions of a Device. For example, a Fan Device can have an associated Tachometer which measures its speed. A Counter is a general purpose Sensor that measures some numerical property of a Device. A Counter value can be cleared, but it never decreases. A Switch Sensor has states, like Open/Close, On/Off, or Up/Down. A Lock has states of Locked/Unlocked. Humidity, Smoke Detection, and Air Flow Sensors measure the equivalent environmental characteristics. A Presence Sensor detects the presence of a Physical Element. A Power Consumption Sensor measures the instantaneous power consumed by a managed element. A Power Production Sensor measures the instantaneous power produced by a managed element such as a power supply or a voltage regulator. A pressure sensor is used to report pressure. Value Map: 0 – Unknown 1 – Other 2 – Temperature 3 – Voltage 4 – Current 5 – Tachometer 6 – Counter 7 – Switch 8 – Lock 9 – Humidity 10 – Smoke Detection 11 – Presence 12 – Air Flow 13 – Power Consumption 14 – Power Production 15 – Pressure 16-32766 – DMTF Reserved 32768-65535 – Vendor Reserved |
| OtherSensorTypeDescription | string | A string describing the Sensor type - used when the SensorType property is set to "Other". |
| PossibleStates | string[] | PossibleStates is an array that enumerates the string outputs of the Sensor. For example, a "Switch" Sensor may output the states "On", or "Off". Another implementation of the Switch may output the states "Open", and "Close". Another example is a NumericSensor supporting thresholds. This Sensor can report the states like "Normal", "Upper Fatal", "Lower Non-Critical", etc. A NumericSensor that does not publish readings and thresholds, but stores this data internally, can still report its states. |
| CurrentState | string | The current state indicated by the Sensor. This is always one of the "PossibleStates". |
| PollingInterval | ulong | The polling interval that the Sensor hardware or the instrumentation uses to determine the current state of the Sensor. Units: Nanoseconds |
| BaseUnits | ushort | The base unit of the values returned by this Sensor. All the values returned by this Sensor are represented in the units obtained by (BaseUnits * 10 raised to the power of the UnitModifier). For example, if BaseUnits is Volts and the UnitModifier is -6, then the units of the values returned are MicroVolts. However, if the RateUnits property is set to a value other than "None", then the units are further qualified as rate units. In the above example, if RateUnits is set to "Per Second", then the values returned by the Sensor are in MicroVolts/Second. The units apply to all numeric properties of the Sensor, unless explicitly overridden by the Units qualifier. ValueMap: 0 – 66 : //0 "Unknown", "Other", "Degrees C", "Degrees F", "Degrees K", "Volts", "Amps", "Watts", "Joules", "Coulombs", //10 "VA", "Nits", "Lumens", "Lux", "Candelas", "kPa", "PSI", "Newtons", "CFM", "RPM", //20 "Hertz", "Seconds", "Minutes", "Hours", "Days", "Weeks", "Mils", "Inches", "Feet", "Cubic Inches", //30 "Cubic Feet", "Meters", "Cubic Centimeters", "Cubic Meters", "Liters", "Fluid Ounces", "Radians", "Steradians", "Revolutions", "Cycles", //40 "Gravities", "Ounces", "Pounds", "Foot-Pounds", "Ounce-Inches", "Gauss", "Gilberts", "Henries", "Farads", "Ohms", //50 "Siemens", "Moles", "Becquerels", "PPM (parts/million)", "Decibels", "DbA", "DbC", "Grays", "Sieverts", "Color Temperature Degrees K", //60 "Bits", "Bytes", "Words (data)", "DoubleWords", "QuadWords", "Percentage", "Pascals" In IPMI v1.5, spec. table 17-15 lists all IPMI sensor units, there are three types of mapping here: 1. Direct mapping, like IPMI unit type 1 “degrees C” maps to CIM BaseUints 2 “Degrees C”. 2. Indirect mapping, like IPMI unit type 20 “microsecond” maps to CIM BaseUnits 21 “Seconds”, then the scale difference -6 should be reflected in property UnitModifier. 3. All other IPMI units map to “Other” (2). |
| UnitModifier | int | The unit multiplier for the values returned by this Sensor. All the values returned by this Sensor are represented in the units obtained by (BaseUnits * 10 raised to the power of the UnitModifier). For example, if BaseUnits is Volts and the Unit Modifier is -6, then the units of the values returned are MicroVolts. However, if the RateUnits property is set to a value other than "None", then the units are further qualified as rate units. In the above example, if RateUnits is set to "Per Second", then the values returned by the Sensor are in MicroVolts/Second. The units apply to all numeric properties of the Sensor, unless explicitly overridden by the Units qualifier. |
| RateUnits | ushort | Specifies if the units returned by this Sensor are rate units. All the values returned by this Sensor are represented in the units obtained by (BaseUnits * 10 raised to the power of the UnitModifier). This is true unless this property (RateUnits) has a value different than "None". For example, if BaseUnits is Volts and the UnitModifier is -6, then the units of the values returned are MicroVolts. But, if the RateUnits property is set to a value other than "None", then the units are further qualified as rate units. In the above example, if RateUnits is set to "Per Second", then the values returned by the Sensor are in MicroVolts/Second. The units apply to all numeric properties of the Sensor, unless explicitly overridden by the Units qualifier. Any implementation of CurrentReading should be qualified with either a Counter or a Gauge qualifier, depending on the characteristics of the sensor being modeled. ValueMap: 0 – None 1 – Per MicroSecond 2 – Per MilliSecond 3 – Per Second 4 – Per Minute 5 – Per Hour 6 – Per Day 7 – Per Week 8 – Per Month 9 – Per Year |
| CurrentReading | int | The current value indicated by the Sensor. |
| NominalReading | int | NominalReading indicates the 'normal' or expected value for the NumericSensor. |
| NormalMax | int | NormalMax provides guidance for the user as to the normal maximum range for the NumericSensor. |
| NormalMin | int | NormalMin provides guidance for the user as to the normal minimum range for the NumericSensor. |
| MaxReadable | int | MaxReadable indicates the largest value of the measured property that can be read by the NumericSensor. |
| MinReadable | int | MinReadable indicates the smallest value of the measured property that can be read by the NumericSensor. |
| Resolution | uint | Resolution indicates the ability of the Sensor to resolve differences in the measured property. The units for this measurement are determined by BaseUnit*UnitModifier/RateUnit. |
| IsLinear | bool | Indicates that the Sensor is linear over its dynamic range. |
| Hysteresis | uint | Indicates the margin built around the thresholds. This margin prevents unnecessary state changes when the Sensor reading may fluctuate very close to its thresholds. This could be due to the Sensor's tolerance/accuracy/resolution or due to environmental factors. Once a threshold is crossed, the state of the Sensor should change. However, the state should not fluctuate between the old and new states unless the Sensor's change in the reading exceeds the hysteresis value. The units for this measurement are determined by BaseUnit*UnitModifier/RateUnit. |
| LowerThresholdNonCritical | int | The Sensor's threshold values specify the ranges (min and max values) for determining whether the Sensor is operating under Normal, NonCritical, Critical, or Fatal conditions. If Current Reading is between LowerThresholdNonCritical and Upper ThresholdNonCritical, then the Sensor is reporting a normal value. If CurrentReading is between LowerThresholdNonCritical and LowerThresholdCritical, then the CurrentState is NonCritical. |
| UpperThresholdNonCritical | int | The Sensor's threshold values specify the ranges (min and max values) for determining whether the Sensor is operating under Normal, NonCritical, Critical, or Fatal conditions. If the CurrentReading is between LowerThresholdNonCritical and UpperThresholdNonCritical, then the Sensor is reporting a normal value. If the CurrentReading is between UpperThreshold NonCritical and UpperThresholdCritical, then the CurrentState is NonCritical. |
| LowerThresholdCritical | int | The Sensor's threshold values specify the ranges (min and max values) for determining whether the Sensor is operating under Normal, NonCritical, Critical, or Fatal conditions. If the CurrentReading is between LowerThresholdCritical and Lower ThresholdFatal, then the CurrentState is Critical. |
| UpperThresholdCritical | int | The Sensor's threshold values specify the ranges (min and max values) for determining whether the Sensor is operating under Normal, NonCritical, Critical, or Fatal conditions. If the CurrentReading is between UpperThresholdCritical and Upper ThresholdFatal, then the CurrentState is Critical. |
| LowerThresholdFatal | int | The Sensor's threshold values specify the ranges (min and max values) for determining whether the Sensor is operating under Normal, NonCritical, Critical, or Fatal conditions. If the CurrentReading is below LowerThresholdFatal, then the Current State is Fatal. |
| UpperThresholdFatal | int | The Sensor's threshold values specify the ranges (min and max values) for determining whether the Sensor is operating under Normal, NonCritical, Critical, or Fatal conditions. If the CurrentReading is above UpperThresholdFatal, then the Current State is Fatal. |
| SupportedThresholds | ushort[] | An array representing the supported thresholds by this Sensor. If no thresholds are supported, this shall be an empty array. ValueMap: 0, 1, 2, 3, 4, 5 Values: LowerThresholdNonCritical, UpperThresholdNonCritical, LowerThresholdCritical, UpperThresholdCritical, LowerThresholdFatal, UpperThresholdFatal |
| EnableThresholds | ushort[] | An array representing the thresholds that are currently enabled for this Sensor. If no thresholds are enabled, this shall be an empty array. ValueMap: 0, 1, 2, 3, 4, 5 Values: LowerThresholdNonCritical, UpperThresholdNonCritical, LowerThresholdCritical, UpperThresholdCritical, LowerThresholdFatal, UpperThresholdFatal |
| SettableThresholds | ushort[] | An array representing the writeable thresholds that are supported for this Sensor. If no thresholds are settable, this shall be an empty array. ValueMap: 0, 1, 2, 3, 4, 5 Values: LowerThresholdNonCritical, UpperThresholdNonCritical, LowerThresholdCritical, UpperThresholdCritical, LowerThresholdFatal, UpperThresholdFatal |

## Enumerated List Clarifications

When implementing this data model with SOAP, all enumerated lists shall be implemented as lower case strings (not numbers), such as "int", "bool", etc.

### DataType

Several fields in the data model contain a free form string with an associated "DataType" field that defines the encoding of the string. String representations of values shall be encoded as the examples below.

| DataType | Encoding Example |
| --- | --- |
| null | <empty string> |
| int | 5376 or -4212 |
| bool | true or false or TRUE or FALSE |
| float | 23532.23 |
| byte | 0xAC |
| string | free form text (single line) |
| text | free form text multi-line |
| bitmap | 1010101010 |
| blob | <base64 encoded> *see RFC 2045 Section 6.8 -- http://www.ietf.org/rfc/rfc2045.txt |
| uint | 743947 (32bit) |
| datetime | <int of seconds since Jan 1, 1970> |
| macaddr | 0a:0b:0c:0d:0e:0f |
| ipaddr | 127.0.0.1 |
| json | {"stringKey": "svalue", "numericKey": 3, "boolKey": true}..... *see JSON Encoding -- http://www.json.org |
| xml | <xml version="1.0">..... *see XML Encoding -- http://www.w3.org/XML/ |

### SeverityLevel

Applies to HealthState, Sensor, SensorValue, LogEntry

| Value | Description |
| --- | --- |
| unknown | Severity can not be determined |
| info (neutral) | This is primarily used for logged events. For instance, if someone changes an IP address, this may not be ok, warning, critical, etc., but it may still be logged, so the severity would most likely be Info or "neutral" severity. |
| ok | Within the scope and context of the element this SeverityLevel is attached to, it is in a "good" state. When used for log entries, this would usually represent a transition from a Critical or Warning state back to an Ok state. |
| warning | An event or state that would require some user action. |
| critical | Represents a failure of some element. |

# SOAP Interface Conventions

## Interfaces

A SOAP interface shall be created for each class in the Dashboard Class Model that has at least one method. In this case, Component and LogEntry have their own SOAP interfaces.

## Method Invocation

All method definitions and invocations shall be implemented such that the InstanceID of the Object is passed as the first parameter. In the case of the Component Class, the InstanceID of "ROOT" will return the First Top Level Object in the Tree of Components.

## Composition AccessMethods

The Component object shall implement the following methods

### Single Instance

Each Ws* interface shall implement the following method.

getInstance(InstanceID): <Class Name>

This method returns a specific instance of the Class.

### Multiple Aggregated Instances

For each Aggregation, the Containing Class shall have a member variable that is the name of the aggregated class. Example: Component::LogEntry[]. However, Instantiation of the Component Class does not imply that Aggregated Instances are filled. The methods below are used to access filled Aggregated Arrays.

The Component Class shall implement:

getAggregatedInstances(InstanceID, ClassAggregateFilter[]): Component

The method will return a Component with member array variables named <ClassName>[] filled in based on the filter specifics of ClassAggregateFilter.

The resultant Component Instance returned will have the following structure:

### Specific Access Methods

To get the instances of a specific type within a Component, the following methods shall be implemented within WsComponent.

get<ClassName>Instances(AggregateFilter): <ClassName>[]

This method returns an array of aggregated instances in the type of the aggregated class.

Example: 
WsComponent::getHealthElementInstances(AggregateFilter): HealthElement[]

### Filter Parameters

The ClassAggregateFilter adds the ClassName to the Aggregate filter to specify the filter for a specific class. The Aggregate Filter parameters are defined in the table below.

AggregateFilter Properties

| Property | Type | Notes |
| --- | --- | --- |
| Depth | int | Recursive Depth This field only applies to getAggregatedInstances and getComponentInstances. It is otherwise ignored. By filling this value, sub-Components shall recursively be returned within their respective parent Components. If the depth does not reach the specified depth, then all objects within the specified depth shall be returned and this case shall not generate an error condition. Value; Meaning. 0; Infinite. 1; Top level only. 2 .. N; Additional Levels Deep. |
| Range | string | Range of Objects to Retrieve This is especially useful for log entries. The Values represented in the table below imply that the dash "-" is taken literally. Example: "1-5" would be the string to request objects 1-5. If there is not an object that represents every member of a range, then the subset (or empty set) that does fall within the range shall be returned. This can happen when paging through object and reaching the end of the list. If you specify objects 1-5 and only 1-3 exist, then records 1-3 shall be returned. If no objects meet the range, then an empty set shall be returned. This case shall not generate an error condition. Range Definition Value; Meaning. <empty>; All. -N; From first record to N. M-N; From M to N. N-; Everything after N. |
| Where | string | JSON encoded string of Property=Values If values are expressed as an array ( eg: [value1,value2] ), the implied logical expression is the same key being equal to any one of the values (or). Each additional Property=value pair implies an "and" expression with the previous defined Property=value pairs Example { "SensorType"=[2,3], "CurrentState"="Normal" } Would return all Sensors where: SensorType = "Temperature" or SensorType = "Voltage" and CurrentState = "Normal". |

## SOAP WSDL Locations

To determine the proper URLs for the WSDL description docs, a Web Service Discovery file shall exist at the following URL:  

http://<CMM IP ADDRESS>/dashboard.disco

The contents of the DISCO file shall be the following with substitutions for the URLs below enclosed in "<>":

<disco: discovery

xmlns:disco="http://schemas.xmlsoap.org/disco/">

xmlns:scl="http://schemas.xmlsoap.org/disco/scl/">

<contractRef

ref="<URL-TO-WsComponent-WSDL>"

docRef="<URL-TO-WsComponent-WSDL-Documentation>"/>

<contractRef

ref="<URL-TO-WsAction-WSDL>"

docRef="<URL-TO-WsAction-WSDL-Documentation>"/>

<contractRef

ref="<URL-TO-WsLogentry-WSDL>"

docRef="<URL-TO-WsLogEntry-WSDL-Documentation>"/>

</disco>

An example would be the following:

<disco: discovery

xmlns:disco="http://schemas.xmlsoap.org/disco/">

xmlns:scl="http://schemas.xmlsoap.org/disco/scl/">

<contractRef 
            ref="http://192.168.200.1/soap/WsComponent.asp?wsdl"

docRef=" http://192.168.200.1/soap/WsComponent.asp?doc"/>

<contractRef

ref=" http://192.168.200.1/soap/WsLogEntry.asp?wsdl"

docRef=" http://192.168.200.1/soap/WsComponent.asp?doc"/>

</disco>

- – SOAP Use Cases

  - Component Method Calls

Perhaps the simplest call to make is a power control feature of a component. In this example, A SOAP call is made to "identify" Blade Server 1 for 60 seconds (in most implementations, this would illuminate a LED).

Pseudo Call:

WsComponent::identify("1",60);  // InstanceID=1, 60 Seconds

Request:

<soapenv:Envelope

xmlns:soapenv="http://schemas.xmlsoap.org/soap/envelope/">

<soapenv:Body soapenv:encodingStyle="http://schemas.xmlsoap.org/soap/encoding/">

<s0:identify

xmlns:soapenc="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns:s0="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent">

<InstanceID xsi:type="xs:string">1</InstanceID>

<Mode xsi:type="xs:unsignedInt">60</Mode>

</s0:identify>

</soapenv:Body>

</soapenv:Envelope>

Response:

<SOAP-ENV:Envelope SOAP-ENV:encodingStyle="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:SOAP-ENV="http://schemas.xmlsoap.org/soap/envelope/"

xmlns:ns1="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"

xmlns:xsd="http://www.w3.org/2001/XMLSchema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns:SOAP-ENC="http://schemas.xmlsoap.org/soap/encoding/">

<SOAP-ENV:Body>

<ns1:identifyResponse>

<identifyResponse xsi:type="xsd:short">0</identifyResponse>

</ns1:identifyResponse>

</SOAP-ENV:Body>

</SOAP-ENV:Envelope>

  - Aggregated Data

The Component class has methods for each associated class. The method name is in the pattern of get<classname>Instances(InstanceID,Filter). So, in this example, we will retrieve the Log Entries at the ROOT Component context. This is how one would get all Log Entries.

Pseudo Call:

Filter = new AggregateFilter(0,"","") // Assuming constructor will populate filter

WsComponent::getLogEntryInstances("ROOT",Filter);

Request:

<soapenv:Envelope

xmlns:soapenv="http://schemas.xmlsoap.org/soap/envelope/">

<soapenv:Body soapenv:encodingStyle="http://schemas.xmlsoap.org/soap/encoding/">

<s0:getLogEntryInstances

xmlns:typens="urn:WsComponent"

xmlns:soapenc="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns:s0="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent">

<InstanceID xsi:type="xs:string">ROOT</InstanceID>

<Filter xsi:type="typens:AggregateFilter">

<Depth xsi:type="xs:int">0</Depth>

<Range xsi:type="xs:string" />

<Where xsi:type="xs:string" />

</Filter>

</s0:getLogEntryInstances>

</soapenv:Body>

</soapenv:Envelope>

Response:

<SOAP-ENV:Envelope SOAP-ENV:encodingStyle="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:SOAP-ENV="http://schemas.xmlsoap.org/soap/envelope/"

xmlns:ns1="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"

xmlns:ns2="urn:WsComponent"

xmlns:SOAP-ENC="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:xsd="http://www.w3.org/2001/XMLSchema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<SOAP-ENV:Body>

<ns1:getLogEntryInstancesResponse>

<getLogEntryInstancesResponse SOAP-ENC:arrayType="ns2:LogEntry[2]" xsi:type="ns2:ArrayOfLogEntry">

<item xsi:type="ns2:LogEntry">

<InstanceID xsi:type="xsd:string">1</InstanceID>

<ComponentInstanceID xsi:type="xsd:string">1</ComponentInstanceID>

<Caption xsi:type="xsd:string">CPU Hot</Caption>

<Closed xsi:type="xsd:boolean">true</Closed>

<DateTime xsi:type="xsd:int">1253133328</DateTime>

<Description xsi:type="xsd:string">CPU Has exceeded upper threshold of 100 degC</Description>

<Severity xsi:type="xsd:string">critical</Severity>

</item>

<item xsi:type="ns2:LogEntry">

<InstanceID xsi:type="xsd:string">2</InstanceID>

<ComponentInstanceID xsi:type="xsd:string">1</ComponentInstanceID>

<Caption xsi:type="xsd:string">Compute Blade 1 Inserted</Caption>

<Closed xsi:type="xsd:boolean">true</Closed>

<DateTime xsi:type="xsd:int">1253133328</DateTime>

<Description xsi:type="xsd:string">CPU Has exceeded upper threshold of 100 degC</Description>

<Severity xsi:type="xsd:string">ok</Severity>

</item>

</getLogEntryInstancesResponse>

</ns1:getLogEntryInstancesResponse>

</SOAP-ENV:Body>

</SOAP-ENV:Envelope>

  - Actions

If a Compute Blade has a single (custom) Action called "KVM" (Keyboard/Video/Mouse), and we wanted to present that to the user and then take input from the user of the mouse mode and the color depth to use in management, we could do the following:

1. Get the Actions Available

Filter = new AggregateFilter(0,"","");
WsComponent::getActionInstances("1","","");

Request:

<soapenv:Envelope

xmlns:soapenv="http://schemas.xmlsoap.org/soap/envelope/">

<soapenv:Body soapenv:encodingStyle="http://schemas.xmlsoap.org/soap/encoding/">

<s0:getActionInstances

xmlns:typens="urn:WsComponent"

xmlns:soapenc="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns:s0="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent">

<InstanceID xsi:type="xs:string">1</InstanceID>

<Filter xsi:type="typens:AggregateFilter">

<Depth xsi:type="xs:int">0</Depth>

<Range xsi:type="xs:string" />

<Where xsi:type="xs:string" />

</Filter>

</s0:getActionInstances>

</soapenv:Body>

</soapenv:Envelope>

Response:

<SOAP-ENV:Envelope SOAP-ENV:encodingStyle="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:SOAP-ENV="http://schemas.xmlsoap.org/soap/envelope/"

xmlns:ns1="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"

xmlns:ns2="urn:WsComponent"

xmlns:SOAP-ENC="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:xsd="http://www.w3.org/2001/XMLSchema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance">

<SOAP-ENV:Body>

<ns1:getActionInstancesResponse>

<getActionInstancesResponse SOAP-ENC:arrayType="ns2:Action[1]" xsi:type="ns2:ArrayOfAction">

<item xsi:type="ns2:Action">

<InstanceID xsi:type="xsd:string">1</InstanceID>

<HelpText xsi:type="xsd:string">Start a Remote Keyboard/Video/Mouse Session</HelpText>

<Caption xsi:type="xsd:string">Start KVM</Caption>

<Name xsi:type="xsd:string">StartKVM</Name>

<ActionType xsi:type="xsd:string">dialog</ActionType>

<URLInfo xsi:type="xsd:string"> </URLInfo>

<ActionParam 
                 SOAP-ENC:arrayType="ns2:ActionParam[2]" xsi:type="ns2:ArrayOfActionParam">

<item xsi:type="ns2:ActionParam">

<DataType xsi:type="xsd:string">int</DataType>

<Default xsi:type="xsd:string">1</Default>

<DisplayType xsi:type="xsd:string">select</DisplayType>

<Caption xsi:type="xsd:string">Mouse Mode</Caption>

<Name xsi:type="xsd:string">MouseMode</Name>

<OptionPrompts xsi:type="xsd:string">Relative,Absolute</OptionPrompts>

<OptionValues xsi:type="xsd:string">1,2</OptionValues>

</item>

<item xsi:type="ns2:ActionParam">

<DataType xsi:type="xsd:string">int</DataType>

<Default xsi:type="xsd:string">1</Default>

<DisplayType xsi:type="xsd:string">select</DisplayType>

<Caption xsi:type="xsd:string">Color Depth</Caption>

<Name xsi:type="xsd:string">ColorDepth</Name>

<OptionPrompts xsi:type="xsd:string">Low (8bit),High (16bit),True (24bit)</OptionPrompts>

<OptionValues xsi:type="xsd:string">8,16,24</OptionValues>

</item>

</ActionParam>

</item>

</getActionInstancesResponse>

</ns1:getActionInstancesResponse>

</SOAP-ENV:Body>

</SOAP-ENV:Envelope>

- The UI would be constructed with the Information from the response above with a button for each Action (in addition to the embedded power control methods within the Component). When the KVM button is pressed, a dialog will be presented requesting input from the user for each ActionParam in the above response.

- Once the data is gathered, then the invokeAction is called on the component using the Action::Name, and data gathered from the user.

Pseudo call:

WsComponent::invokeAction('1','StartKVM','{"MouseMode":1,"ColorDepth":24}')

Request:

<soapenv:Envelope

xmlns:soapenv="http://schemas.xmlsoap.org/soap/envelope/">

<soapenv:Body soapenv:encodingStyle="http://schemas.xmlsoap.org/soap/encoding/">

<s0:invokeAction

xmlns:soapenc="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns:s0="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent">

<InstanceID xsi:type="xs:string">1</InstanceID>

<ActionName xsi:type="xs:string">StartKVM</ActionName>

<ActionParameters xsi:type="xs:string">{"MouseMode":1,"ColorDepth":24}</ActionParameters>

</s0:invokeAction>

</soapenv:Body>

</soapenv:Envelope>

Response:

<SOAP-ENV:Envelope SOAP-ENV:encodingStyle="http://schemas.xmlsoap.org/soap/encoding/"

xmlns:SOAP-ENV="http://schemas.xmlsoap.org/soap/envelope/"

xmlns:ns1="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"

xmlns:xsd="http://www.w3.org/2001/XMLSchema"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xmlns:SOAP-ENC="http://schemas.xmlsoap.org/soap/encoding/">

<SOAP-ENV:Body>

<ns1:invokeActionResponse>

<invokeActionResponse xsi:type="xsd:short">0</invokeActionResponse>

</ns1:invokeActionResponse>

</SOAP-ENV:Body>

</SOAP-ENV:Envelope>

5. The Service then executes whatever is necessary for performing this action.

  - Composite Aggregated Data

This service also has a mechanism to retrieve all data in a "tree" format where the aggregated data is composed as a part of the Component instance (as array properties of the Component). The getAggregatedInstances() method is used for this purpose. This method will return a Component that contains the selected aggregated data as well as sub-Components and the selected aggregated data of those as well. The call below will return a top-level component (such as a Chassis), along with second level components (such as Blades and Switches). Additionally, all InfoElements and HealthStatuses would be returned in the tree as well.

Pseudo Call:

ClassFilter = array(

new ClassAggregateFilter("Component",2,"",""), // 2 Levels Deep

new ClassAggregateFilter("InfoElement",0,"",""),

new ClassAggregateFilter("HealthStatus",0,"","")

);

WsComponent::getAggregateInstances("ROOT",ClassFilter);

- -WSDLs

WsComponent

<?xml version="1.0" encoding="utf-8"?>
<definitions xmlns="http://schemas.xmlsoap.org/wsdl/" name="WsComponent"
targetNamespace="urn:WsComponent" xmlns:typens="urn:WsComponent"
xmlns:xsd="http://www.w3.org/2001/XMLSchema"
xmlns:soap="http://schemas.xmlsoap.org/wsdl/soap/"
xmlns:soapenc="http://schemas.xmlsoap.org/soap/encoding/"
xmlns:wsdl="http://schemas.xmlsoap.org/wsdl/">
 <types>
   <xsd:schema xmlns="http://www.w3.org/2001/XMLSchema"
   targetNamespace="urn:WsComponent">
     <xsd:complexType name="ArrayOfunsignedShort">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="xsd:unsignedShort[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfstring">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="xsd:string[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfSensor">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="typens:Sensor[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfInfoElement">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="typens:InfoElement[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfHealthElement">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="typens:HealthElement[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfLogEntry">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="typens:LogEntry[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfActionParam">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="typens:ActionParam[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfAction">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="typens:Action[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfComponent">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="typens:Component[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="ArrayOfClassAggregateFilter">
       <xsd:complexContent>
         <xsd:restriction base="soapenc:Array">
           <xsd:attribute ref="soapenc:arrayType"
           wsdl:arrayType="typens:ClassAggregateFilter[]" />
         </xsd:restriction>
       </xsd:complexContent>
     </xsd:complexType>
     <xsd:complexType name="Component">
       <xsd:all>
         <xsd:element name="InstanceID" type="xsd:string" />
         <xsd:element name="ComponentType" type="xsd:string" />
         <xsd:element name="ComponentTypeDescription" type="xsd:string" />
         <xsd:element name="Caption" type="xsd:string" />
         <xsd:element name="LocatorURL" type="xsd:string" />
         <xsd:element name="HealthState" type="typens:HealthState" />
         <xsd:element name="OperationalStatus" type="xsd:string" />
         <xsd:element name="OperationalStatusDescription" type="xsd:string" />
         <xsd:element name="PowerControlCapabilities"
         type="typens:ArrayOfunsignedShort" />
         <xsd:element name="TypeOrdinal" type="xsd:int" />
         <xsd:element name="IdentifyState" type="xsd:bool" />
         <xsd:element name="Sensor" type="typens:ArrayOfSensor" />
         <xsd:element name="InfoElement" type="typens:ArrayOfInfoElement" />
         <xsd:element name="HealthElement"
         type="typens:ArrayOfHealthElement" />
         <xsd:element name="LogEntry" type="typens:ArrayOfLogEntry" />
         <xsd:element name="Action" type="typens:ArrayOfAction" />
         <xsd:element name="Component" type="typens:ArrayOfComponent" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="HealthState">
       <xsd:all>
         <xsd:element name="Severity" type="xsd:int" />
         <xsd:element name="Caption" type="xsd:string" />
         <xsd:element name="Description" type="xsd:string" />
         <xsd:element name="RecommendedAction" type="xsd:string" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="Sensor">
       <xsd:all>
         <xsd:element name="Caption" type="xsd:string" />
         <xsd:element name="Accuracy" type="xsd:int" />
         <xsd:element name="SensorType" type="xsd:unsignedShort" />
         <xsd:element name="OtherSensorTypeDescription" type="xsd:string" />
         <xsd:element name="PossibleStates" type="typens:ArrayOfstring" />
         <xsd:element name="CurrentState" type="xsd:string" />
         <xsd:element name="PollingInterval" type="xsd:unsignedLong" />
         <xsd:element name="BaseUnits" type="xsd:unsignedShort" />
         <xsd:element name="UnitModifier" type="xsd:int" />
         <xsd:element name="RateUnits" type="xsd:unsignedShort" />
         <xsd:element name="CurrentReading" type="xsd:int" />
         <xsd:element name="NominalReading" type="xsd:int" />
         <xsd:element name="NormalMax" type="xsd:int" />
         <xsd:element name="NormalMin" type="xsd:int" />
         <xsd:element name="MaxReadable" type="xsd:int" />
         <xsd:element name="MinReadable" type="xsd:int" />
         <xsd:element name="Resolution" type="xsd:unsignedInt" />
         <xsd:element name="IsLinear" type="xsd:boolean" />
         <xsd:element name="Hysteresis" type="xsd:unsignedInt" />
         <xsd:element name="LowerThresholdNonCritical" type="xsd:int" />
         <xsd:element name="UpperThresholdNonCritical" type="xsd:int" />
         <xsd:element name="LowerThresholdCritical" type="xsd:int" />
         <xsd:element name="UpperThresholdCritical" type="xsd:int" />
         <xsd:element name="LowerThresholdFatal" type="xsd:int" />
         <xsd:element name="UpperThresholdFatal" type="xsd:int" />
         <xsd:element name="SupportedThresholds" type="xsd:unsignedShort" />
         <xsd:element name="EnableThresholds"
         type="typens:ArrayOfunsignedShort" />
         <xsd:element name="SettableThresholds" type="xsd:unsignedShort" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="InfoElement">
       <xsd:all>
         <xsd:element name="DataType" type="xsd:int" />
         <xsd:element name="Caption" type="xsd:string" />
         <xsd:element name="Name" type="xsd:string" />
         <xsd:element name="Value" type="xsd:string" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="HealthElement">
       <xsd:all>
         <xsd:element name="ComponentInstanceID" type="xsd:string" />
         <xsd:element name="HealthStatus" type="typens:HealthState" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="LogEntry">
       <xsd:all>
         <xsd:element name="InstanceID" type="xsd:string" />
         <xsd:element name="ComponentInstanceID" type="xsd:string" />
         <xsd:element name="Caption" type="xsd:string" />
         <xsd:element name="Closed" type="xsd:boolean" />
         <xsd:element name="DateTime" type="xsd:int" />
         <xsd:element name="Description" type="xsd:string" />
         <xsd:element name="Severity" type="xsd:unsignedShort" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="Action">
       <xsd:all>
         <xsd:element name="InstanceID" type="xsd:string" />
         <xsd:element name="HelpText" type="xsd:string" />
         <xsd:element name="Caption" type="xsd:string" />
         <xsd:element name="Name" type="xsd:string" />
         <xsd:element name="ActionType" type="xsd:int" />
         <xsd:element name="URLInfo" type="xsd:string" />
         <xsd:element name="ActionParam" type="typens:ArrayOfActionParam" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="ActionParam">
       <xsd:all>
         <xsd:element name="DataType" type="xsd:int" />
         <xsd:element name="Default" type="xsd:string" />
         <xsd:element name="DisplayType" type="xsd:int" />
         <xsd:element name="Caption" type="xsd:string" />
         <xsd:element name="Name" type="xsd:string" />
         <xsd:element name="OptionPrompts" type="xsd:string" />
         <xsd:element name="OptionValues" type="xsd:string" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="ClassAggregateFilter">
       <xsd:all>
         <xsd:element name="ClassName" type="xsd:string" />
         <xsd:element name="Depth" type="xsd:int" />
         <xsd:element name="Limit" type="xsd:int" />
         <xsd:element name="Where" type="xsd:string" />
       </xsd:all>
     </xsd:complexType>
     <xsd:complexType name="AggregateFilter">
       <xsd:all>
         <xsd:element name="Depth" type="xsd:int" />
         <xsd:element name="Limit" type="xsd:int" />
         <xsd:element name="Where" type="xsd:string" />
       </xsd:all>
     </xsd:complexType>
   </xsd:schema>
 </types>
 <message name="start">
   <part name="InstanceID" type="xsd:string" />
 </message>
 <message name="startResponse">
   <part name="startResponse" type="xsd:short" />
 </message>
 <message name="stop">
   <part name="InstanceID" type="xsd:string" />
 </message>
 <message name="stopResponse">
   <part name="stopResponse" type="xsd:short" />
 </message>
 <message name="reset">
   <part name="InstanceID" type="xsd:string" />
 </message>
 <message name="resetResponse">
   <part name="resetResponse" type="xsd:short" />
 </message>
 <message name="powerCycle">
   <part name="InstanceID" type="xsd:string" />
 </message>
 <message name="powerCycleResponse">
   <part name="powerCycleResponse" type="xsd:short" />
 </message>
 <message name="gracefulShutdown">
   <part name="InstanceID" type="xsd:string" />
 </message>
 <message name="gracefulShutdownResponse">
   <part name="gracefulShutdownResponse" type="xsd:short" />
 </message>
 <message name="invokeAction">
   <part name="InstanceID" type="xsd:string" />
   <part name="ActionName" type="xsd:string" />
   <part name="Parameters" type="xsd:string" />
 </message>
 <message name="invokeActionResponse">
   <part name="invokeActionResponse" type="xsd:short" />
 </message>
 <message name="getInstance">
   <part name="InstanceID" type="xsd:string" />
 </message>
 <message name="getInstanceResponse">
   <part name="getInstanceResponse" type="typens:Component" />
 </message>
 <message name="getAggregateInstances">
   <part name="InstanceID" type="xsd:string" />
   <part name="Filter" type="typens:ArrayOfClassAggregateFilter" />
 </message>
 <message name="getAggregateInstancesResponse">
   <part name="getAggregateInstancesResponse" type="typens:Component" />
 </message>
 <message name="getComponentInstances">
   <part name="InstanceID" type="xsd:string" />
   <part name="Filter" type="typens:AggregateFilter" />
 </message>
 <message name="getComponentInstancesResponse">
   <part name="getComponentInstancesResponse"
   type="typens:ArrayOfComponent" />
 </message>
 <message name="getInfoElementInstances">
   <part name="InstanceID" type="xsd:string" />
   <part name="Filter" type="typens:AggregateFilter" />
 </message>
 <message name="getInfoElementInstancesResponse">
   <part name="getInfoElementInstancesResponse"
   type="typens:ArrayOfInfoElement" />
 </message>
 <message name="getHealthElementInstances">
   <part name="InstanceID" type="xsd:string" />
   <part name="Filter" type="typens:AggregateFilter" />
 </message>
 <message name="getHealthElementInstancesResponse">
   <part name="getHealthElementInstancesResponse"
   type="typens:ArrayOfHealthElement" />
 </message>
 <message name="getLogEntryInstances">
   <part name="InstanceID" type="xsd:string" />
   <part name="Filter" type="typens:AggregateFilter" />
 </message>
 <message name="getLogEntryInstancesResponse">
   <part name="getLogEntryInstancesResponse" type="typens:ArrayOfLogEntry" />
 </message>
 <message name="getActionInstances">
   <part name="InstanceID" type="xsd:string" />
   <part name="Filter" type="typens:AggregateFilter" />
 </message>
 <message name="getActionInstancesResponse">
   <part name="getActionInstancesResponse" type="typens:ArrayOfAction" />
 </message>
 <message name="getSensorInstances">
   <part name="InstanceID" type="xsd:string" />
   <part name="Filter" type="typens:AggregateFilter" />
 </message>
 <message name="getSensorInstancesResponse">
   <part name="getSensorInstancesResponse" type="typens:ArrayOfSensor" />
 </message>
 <portType name="WsComponentPort">
   <operation name="start">
     <documentation>Start will power on the Component. If the component is
     already on, this method will have no effect. Returns: 0 for success,
     Positive Number as an Error Code</documentation>
     <input message="typens:start" />
     <output message="typens:startResponse" />
   </operation>
   <operation name="stop">
     <documentation></documentation>
     <input message="typens:stop" />
     <output message="typens:stopResponse" />
   </operation>
   <operation name="reset">
     <documentation></documentation>
     <input message="typens:reset" />
     <output message="typens:resetResponse" />
   </operation>
   <operation name="powerCycle">
     <documentation></documentation>
     <input message="typens:powerCycle" />
     <output message="typens:powerCycleResponse" />
   </operation>
   <operation name="gracefulShutdown">
     <documentation></documentation>
     <input message="typens:gracefulShutdown" />
     <output message="typens:gracefulShutdownResponse" />
   </operation>
   <operation name="invokeAction">
     <documentation></documentation>
     <input message="typens:invokeAction" />
     <output message="typens:invokeActionResponse" />
   </operation>
   <operation name="getInstance">
     <documentation>Empty String will get Top Level Component</documentation>
     <input message="typens:getInstance" />
     <output message="typens:getInstanceResponse" />
   </operation>
   <operation name="getAggregateInstances">
     <documentation>Populates Aggregated Member Arrays: Valid arrays
     are:</documentation>
     <input message="typens:getAggregateInstances" />
     <output message="typens:getAggregateInstancesResponse" />
   </operation>
   <operation name="getComponentInstances">
     <documentation></documentation>
     <input message="typens:getComponentInstances" />
     <output message="typens:getComponentInstancesResponse" />
   </operation>
   <operation name="getInfoElementInstances">
     <documentation></documentation>
     <input message="typens:getInfoElementInstances" />
     <output message="typens:getInfoElementInstancesResponse" />
   </operation>
   <operation name="getHealthElementInstances">
     <documentation></documentation>
     <input message="typens:getHealthElementInstances" />
     <output message="typens:getHealthElementInstancesResponse" />
   </operation>
   <operation name="getLogEntryInstances">
     <documentation></documentation>
     <input message="typens:getLogEntryInstances" />
     <output message="typens:getLogEntryInstancesResponse" />
   </operation>
   <operation name="getActionInstances">
     <documentation></documentation>
     <input message="typens:getActionInstances" />
     <output message="typens:getActionInstancesResponse" />
   </operation>
   <operation name="getSensorInstances">
     <documentation></documentation>
     <input message="typens:getSensorInstances" />
     <output message="typens:getSensorInstancesResponse" />
   </operation>
 </portType>
 <binding name="WsComponentBinding" type="typens:WsComponentPort">
   <soap:binding style="rpc"
   transport="http://schemas.xmlsoap.org/soap/http" />
   <operation name="start">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="stop">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="reset">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="powerCycle">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="gracefulShutdown">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="invokeAction">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getInstance">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getAggregateInstances">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getComponentInstances">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getInfoElementInstances">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getHealthElementInstances">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getLogEntryInstances">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getActionInstances">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getSensorInstances">
     <soap:operation soapAction="urn:WsComponentAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsComponent"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
 </binding>
 <service name="WsComponent">
   <documentation></documentation>
   <port name="WsComponentPort" binding="typens:WsComponentBinding">
     <soap:address location="http://192.168.56.101/soap/?object=WsComponent" />
   </port>
 </service>
</definitions>

WsLogEntry

<?xml version="1.0" encoding="utf-8"?>
<definitions xmlns="http://schemas.xmlsoap.org/wsdl/" name="WsLogEntry"
targetNamespace="urn:WsLogEntry" xmlns:typens="urn:WsLogEntry"
xmlns:xsd="http://www.w3.org/2001/XMLSchema"
xmlns:soap="http://schemas.xmlsoap.org/wsdl/soap/"
xmlns:soapenc="http://schemas.xmlsoap.org/soap/encoding/"
xmlns:wsdl="http://schemas.xmlsoap.org/wsdl/">
 <types>
   <xsd:schema xmlns="http://www.w3.org/2001/XMLSchema"
   targetNamespace="urn:WsLogEntry">
     <xsd:complexType name="LogEntry">
       <xsd:all>
         <xsd:element name="InstanceID" type="xsd:string" />
         <xsd:element name="ComponentInstanceID" type="xsd:string" />
         <xsd:element name="Caption" type="xsd:string" />
         <xsd:element name="Closed" type="xsd:boolean" />
         <xsd:element name="DateTime" type="xsd:int" />
         <xsd:element name="Description" type="xsd:string" />
         <xsd:element name="Severity" type="xsd:unsignedShort" />
       </xsd:all>
     </xsd:complexType>
   </xsd:schema>
 </types>
 <message name="close">
   <part name="InstanceID" type="xsd:string" />
 </message>
 <message name="closeResponse">
   <part name="closeResponse" type="xsd:unsignedShort" />
 </message>
 <message name="getInstance">
   <part name="InstanceID" type="xsd:string" />
 </message>
 <message name="getInstanceResponse">
   <part name="getInstanceResponse" type="typens:LogEntry" />
 </message>
 <portType name="WsLogEntryPort">
   <operation name="close">
     <documentation></documentation>
     <input message="typens:close" />
     <output message="typens:closeResponse" />
   </operation>
   <operation name="getInstance">
     <documentation></documentation>
     <input message="typens:getInstance" />
     <output message="typens:getInstanceResponse" />
   </operation>
 </portType>
 <binding name="WsLogEntryBinding" type="typens:WsLogEntryPort">
   <soap:binding style="rpc"
   transport="http://schemas.xmlsoap.org/soap/http" />
   <operation name="close">
     <soap:operation soapAction="urn:WsLogEntryAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsLogEntry"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsLogEntry"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
   <operation name="getInstance">
     <soap:operation soapAction="urn:WsLogEntryAction" />
     <input>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsLogEntry"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </input>
     <output>
       <soap:body use="encoded"
       namespace="urn:http://192.168.56.101:80/soap/index.php?object=WsLogEntry"
       encodingStyle="http://schemas.xmlsoap.org/soap/encoding/" />
     </output>
   </operation>
 </binding>
 <service name="WsLogEntry">
   <documentation></documentation>
   <port name="WsLogEntryPort" binding="typens:WsLogEntryBinding">
     <soap:address location="http://192.168.56.101/soap/?object=WsLogEntry" />
   </port>
 </service>
</definitions>